# LongDriver Installation Guide

## Self-Balancing Two-Wheel Vehicle

Contents

LongDriver Installation Guide	1

Self-Balancing Two-Wheel Vehicle	1

1. Component List	2

Frame Components	2

Electronics	2

2. Required Tools	6

3. Assembly Steps	6

Connection diagram	6

4. Electrical Connections	7

Step 1: Power System	7

Step 2: Motor Connections	7

Step 3: Sensor Connections	7

Step 4: Remote Control	8

5. Software Setup	8

Step 1: Arduino IDE Setup	8

Step 2: Code Upload	8

Step 3: Initial Configuration	8

6. Testing and Calibration	9

Step 1: System Check	9

Step 2: Balance Testing	9

Step 3: Remote Control Test	9

Important Notes	9

# 1. Component List

### Frame Components

- 2 wheels (16cm diameter)

- Plastic/aluminum rectangular frame (20x30cm)

- 8mm steel axle

- 608ZZ bearings set

- Nuts and bolts set

- Mounting brackets

### Electronics

- Arduino Mega/Uno board

Figure 1. Arduino Mega/Uno board

- MPU6050 gyroscope/accelerometer

Figure 2. MPU6050 gyroscope/accelerometer

- 2 DC motors 12V (150-300 RPM)

Figure 3. 2 DC motors 12V (150-300 RPM)

- L298N motor driver

Figure 4 L298N

- RF24L01 remote module

Figure 5. RF24L01 remote module

- Battery connectors

Figure 6 Battery

- Motor wheel

Figure 7. wheel

# 2. Required Tools

- Phillips and flathead screwdrivers

- Pliers

- Wire cutters

- Soldering iron and solder

- Electrical tape

- Wrench set

- Caliper

- Multimeter

- Heat shrink tubing

- Cable ties

# 3. Assembly Steps

## Connection diagram

Figure 8. Connection diagram

### Step 1: Frame Assembly

- Base Frame Preparation

  - Clean all frame components

  - Lay out the base plate (20x30cm)

  - Mark mounting points for:

    - Motors

    - Battery compartment

    - Electronics board

    - Sensor mounting

- Wheel Assembly

  - Install bearings into wheel hubs

  - Insert 8mm axle through bearings

  - Secure wheels with lock nuts

  - Check wheel rotation smoothness

- Motor Mounting

  - Attach motor brackets to frame

  - Align motors with wheel axles

  - Secure motors using M3 bolts

  - Verify motor shaft alignment

### Step 4: Electronics Mounting

1  Control Board Installation

Arduino Mounting:

- Position in center of frame

- Use mounting spacers

- Secure with M3 screws

2  Sensor Installation

MPU6050 Mounting:

- Mount at frame center

- Ensure horizontal level

- Use double-sided tape/screws

3  Motor Driver Installation

L298N Mounting:

- Mount near motors

- Allow ventilation space

- Secure with screws

# 4. Electrical Connections

### Step 1: Power System

Battery → Switch → L298N

L298N → Arduino (5V)

### Step 2: Motor Connections

L298N to Motors:

OUT1, OUT2 → Left Motor

OUT3, OUT4 → Right Motor

### Step 3: Sensor Connections

MPU6050 to Arduino:

VCC → 5V

GND → GND

SDA → A4

SCL → A5

### Step 4: Remote Control

RF24L01 to Arduino:

VCC → 3.3V

GND → GND

CE → Pin 7

CSN → Pin 8

SCK → Pin 13

MOSI → Pin 11

MISO → Pin 12

MISO → Pin 12

# 5. Software Setup

### Step 1: Arduino IDE Setup

- Download and install Arduino IDE

- Install required libraries:

- Wire.h

- MPU6050.h

- RF24.h

- PID_v1.h

### Step 2: Code Upload

- Download project code

- Configure board settings:

Board: Arduino Mega

Processor: ATmega2560

Port: COM[X] (select correct port)

- Upload code:

  - Open Arduino IDE

  - Load project file

  - Verify code

  - Upload to board

### Step 3: Initial Configuration

- MPU6050 Calibration:

void calibrateSensor() {

// Place vehicle upright

// Run calibration routine

// Save offset values

}

- PID Tuning:

double Kp = 15;

double Ki = 0.5;

double Kd = 0.8;

# 6. Testing and Calibration

### Step 1: System Check

- Power up system

- Check LED indicators

- Verify sensor readings

### Step 2: Balance Testing

- Hold vehicle upright

- Power on

- Slowly release

- Observe self-balancing

### Step 3: Remote Control Test

- Power on remote

- Test basic movements:

  - Forward/Backward

  - Left/Right turns

  - Emergency stop

# Important Notes

- Double-check all connections before power-up

- Label all wires for maintenance

- Keep detailed assembly photos

- Start with conservative PID values

- Regular maintenance schedule:

  - Check battery voltage

  - Tighten all screws

  - Clean wheel bearings

  - Update software as needed